# APPLICATION FOR APPLYING CORRECTORS WITH COUPLED ERROR TO TOLERANCE ANALYSIS AND ERROR BUDGETS

# Tyler Seawright

M.S. Mechanical Engineering

ERROR BUDGETS AND WHY THEY ARE FEARED

Error budgets are one of the last requests the typical engineer wants to hear and for good reason. We associate “budget” with not having enough or having to decide what to do with a scarce resource. However, without a good error budget, machine performance is impossible to guarantee.

Engineers design incredibly complex systems and with each, it is common to create an entirely new error budget. There is an ocean of details needed to model basic cases and the old method of using Excel spreadsheets is tedious to set up, unstable, and completely unflexible. There is no clear method to capture coupled error motion from applying an error correction.

ERROR BUDGET DEFINITION

Error budgets apportion a total error to identified sources to drive design decisions. A good error budget identifies the sources the system is most sensitive to. With an understanding of corrector coupled error, the error model is more realistic. The concept of an error budget stems from [1].

ABSTRACT PURPOSE

This abstract is intended to serve as a description of an open-source error budget application that includes a workflow to model complex systems with minimal setup time. In this abstract, a sample application will be presented highlighting the capabilities of the application and describe the workflow. The vector transform methods are from rotation matrices described by [2] and a new method for capturing coupled errors propagated from utilizing correctors. A built-in Montecarlo determines the average plus N-sigma error. This application does not focus on distinguishing types of error sources other than mechanical, but they are easily included if a sensitivity is provided. The equations to convert HTM to positions and angles is from [3].

ERROR BUDGET WORKFLOW

DEVELOP MATH MODEL

The math model represents the connection between parts of the error budget. This may include multiple vector paths that are stacked, definition of correctors, and sensitivities within the system. In the script, the math model is written as a comment in the header before implementing into the main function.

INPUT VECTOR LOOPS, ERROR TERMS, AND CORRECTORS

In a screenshot, a sketch of the vector path to each part interface is helpful to organize the model. FIGURE 1 depicts a sample vector path and the excel spreadsheet representing the inputs.

[Will be added in long abstract]

FIGURE 1. Math model for tolerance analysis used in sample application.

The model is input into a spreadsheet as a list of 6 DOF vectors and relative errors. Since errors are relative, they may be input as the part tolerances as they are local to the part coordinate system. Each type of input is tagged to make the spreadsheet dynamic for MATLAB to parse as it becomes complex.

A corrector must be defined with a vector from the corrector to a point of interest and must contain a vector describing the resolution of the corrector per axis of correction.

REVIEW RESULTS

Results are output to the spreadsheet where specified. The script is fully customizable and works well as a function for building complex systems. The script prints results, plots histograms, and has a function for visualizing error and directions using plots of coordinate systems depicted in FIGURE 2.

[Will be added in long abstract]

FIGURE 2. Sample output plots.

ERROR MODEL

VECTOR LOOP ERRORS USING HTM

The relative error between two points in six degrees of freedom is calculated by performing an inverse matrix multiplication of the nominal and errored absolute HTMs representing vector paths between two coordinate systems. Eq. 1 solves the nominal vector transform, Eq. 2 solves the absolute errored transform, and Eq. 3 solves the relative error transform in 6 DOF between the points of interest modeled by a coordinate system.

T0n=T01*T12*…*Tn-1n		(eq. 1)

Te0n=E01T01*E12T12*…*En-1nTn-1n(eq. 2)

Re0n=T0n\Te0n			(eq. 3)

CORRECTOR ERROR PROPAGATION

Correctors pose a challenge in precision engineering because when they are used, there may be coupled motion, which is difficult to quantify for random system error. The coupled motion to move the system from an arbitrary position to the corrected position is applied.

The corrector is applied by solving the corrector difference matrix which is the HTM representing how far the point of interest is from the corrector on the axes of compensation. The inverse corrector difference transform is then applied the corrector vector to point of interest and then to the absolute position HTM to rotate to the corrector position and propagate the parasitic rotation to the coupled axes. The steps are described in FIGURE 3.

[Will be added in long abstract]

FIGURE 3. Flowchart applying correctors to solve relative error at a point of interest.

MODEL VALIDATION

The model was validated by testing a few cases in which an error was applied to a vector and then corrected in 3D and compared to manually calculated tolerance stacks. A sample test case is introducing an error into a vector and compensating for it described in FIGURE 4.

[Will be added in long abstract]

FIGURE 4. Visualization of coordinate transform error for sample test case.

SAMPLE APPLICATION: OPTICAL ALIGNMENT

SYSTEM OVERVIEW AND REQUIREMENTS

The sample system is an axial-symmetric mirror that reflects a collimated beam to hit a target a specific angle and position. The engineer does not know if the mirror can be fixed or requires error correction. The beam is aligned first to a tooling target within 0.05mm before the mirror is inserted into the assembly to direct the beam to a new target. The angle of the beam must fall within 100mdeg at the target with respect to the tooling target. Given the beam is 4mm diameter and clear aperture (CA) of the mirror is 5mm. Because of a nearby beam, a shift in Z by 1mm causes clipping. If an error corrector is to be used, it must satisfy the requirements. TABLE 1. outlines the sample application requirements derived from the sample optical requirements.

| Requirement | Value | Unit |
| --- | --- | --- |
| Mirror Centration (x,y,z) | 0.45 0.45 1.0 | mm mm mm |
| Mirror Orientation (Tx, Ty, Tz) | 1.74 1.74 20 | mrad mrad mrad |

TABLE 1. Sample requirement table. All requirements are in the target coordinate system relative to tooling target.

ERROR MODEL

To be added in Long-Abstract

MODEL OUTPUTS

To be added in Long-Abstract

CONCLUSION

To be added in Long-Abstract

REFERENCES

- Joint Committee for Guides in Metrology. Evaluation of measurement data – Guide to the expression of uncertainty in measurement. 2008.

- A. H. Slocum, Precision Machine Design. Prentice-Hall, Inc., 1992.

- Duenner, Andrew C. An Error Budgeting Framework for Engineering Design. Massachusetts Institute of Technology. 2019.